# SILT FENCE

What is it? 
 	This is a temporary barrier made of woven wire and fabric filter cloth that is used to catch sediment-laden runoff from small areas of disturbed soil such as following a fire. Silt fences are easy to construct, and materials are available from hardware stores, nurseries, and lumberyards.

Silt fences are easy to construct, and materials are available from hardware stores, nurseries, and lumberyards.

When is it used? 
 	Silt fences are used for specific situations. Major considerations are slope, slope length, and the amount of drainage area from which the fence will catch runoff. Here are some design considerations:

| Slope Steepness | Maximum Slope Length |
| --- | --- |
| 2:1 = 50% | 50 feet |
| 3:1 = 33% | 75 feet |
| 4:1 = 25% | 125 feet |
| 5:1 = 20% | 175 feet |
| <5:1 = <20% | 200 feet |
|  | *For longer slopes, add additional silt fences. |

Drainage Area: 
The area that contributes runoff to be caught by the silt fence should not be greater than 1/2 acre for 100 feet of fence.

Type of Runoff: 
 	Silt fences are designed to catch runoff that is in the form of "sheet flow" and not "concentrated flow." Sheet flow differs from concentrated flow in that the runoff is spread evenly over the ground surface (like a sheet) rather than concentrated in small rills or gullies.

Methods and Materials:

Fence Posts: 
 	Posts should be at least 36 inches long. Wood posts should be of hardwood with a minimum cross sectional area of 3 inches. Steel posts should be standard "T" or "U" section and should weigh no less than 1 pound per linear foot.

July 2002 Arizona